- 2000-01-0862

Regulated and Benzene Emissions of In-Use

Two-Stroke Mopeds and Motorcycles

M.V. Prati, M. Rapone, N. Violetti

- Istituto Motori CNR  – viale Marconi, 8  80125 Napoli ITALY

- R. Mercogliano, R. Trerè

- AgipPetroli – via Laurentina 449  00142 Roma ITALY

Copyright © 1998 Society of Automotive Engineers, Inc.

- ABSTRACT

The attention on emissions of two-wheelers has been poor in the past, but today in countries with a large two-wheeler population it gives a significant contribution to aggregate emissions. In this paper the results obtained on a fleet composed by 22 two-stroke motorcycles (including mopeds) are presented. Sixteen in-use mopeds and six 125 cm³ motorcycles have been tested over ECE 47 and ECE 40 driving cycles respectively.

Regulated emissions (CO, HC, NOx), carbon dioxide, benzene and fuel consumption have been evaluated by fueling motorcycles with two different gasoline formulations. One gasoline was a commercial Italian leaded gasoline with 1% benzene content; the other was a lower benzene and aromatics content gasoline. Benzene emissions decreased according to benzene content of gasoline.

- INTRODUCTION

The widespread use of motorized two-wheelers is an increasing phenomenon in less developed, newly industrialized and industrialized countries. The use of motorcycles shows advantages with respect to other motorized vehicles regarding road congestion and parking charges.

In Europe the motorcycles fleet has been estimated in about 10 million vehicles in 1997 [1]. The Italian percentage of this market is around 27%.

Two-wheelers can be differentiated into two classes: motor cycles <50 cm³ cylinder capacity (two stroke only), named mopeds, and motor cycles >50 cm³ cylinder capacity (two and four stroke). In particular those below 125 cc represent about 1/3 of the motorcycles sold in Europe in 1997 and they are mainly used in short trips (home-to-work or home-to-school) in urban areas. Their annual kilometrage is low between 5000-7500 km.

In Europe the Directive 97/24/EC, entered into force in 17th June 1999, established limit values and procedures to evaluate motorcycles emissions. It expects two phases: the first, named EURO 1, takes place during a period of 24 months from the adoption of the Directive; the second, named EURO 2, needs to take place at least three years after its entering into force. For the EURO 2 phase even tighter limits have been defined. The emission limits are schematized in the following tables 1 and 2 for mopeds and motorcycles respectively. The motorcycles are required to meet emission limits over the ECE 40 driving cycle and mopeds over the ECE 47 one.

|  | CO (g/km) | HC + NOx (g/km) |
| --- | --- | --- |
| EURO 1 | 6 | 3 |
| EURO 2 | 1 | 1.2 |

Table 1: Mopeds emission limit values.

|  | CO (g/km) | HC (g/km) | NOx (g/km) |
| --- | --- | --- | --- |
| EURO 1 (two-stroke) | 8 | 4 | 0.1 |
| EURO 1 (four-stroke) | 13 | 3 | 0.3 |

Table 2: Motorcycles emission limit values.

The most known disadvantages of two-stroke engines are high carbon monoxide and unburned hydrocarbons emissions. It’s well known that two-stroke engine without catalytic converter presents a composition of unburned hydrocarbons characterized mainly by species typical of pure fuel [2]. An increasing attention is actually put on the problem of benzene levels in urban areas, because of its high toxicity. In particular in Italian cities, it is estimated that two-wheelers are responsible of about the 30% of the total benzene emissions [3].

Benzene is, indeed, one of the most critical pollutant in Italian urban areas, taking into consideration both the recent occurrence of some exeedances of benzene concentration limit in air (10 g/m3, annual mean) and the future evolution of Air Quality limits in the European Union (EU), where a more stringent limit should be mandatory in year 2010.

One of the objectives of this experimental work was to collect information regarding the emissions of in-use motorcycles, in particular with reference to benzene emission levels. In order to verify if to a decreasing of benzene gasoline content corresponded an equivalent decreasing of benzene emission values, two different gasoline, in terms of benzene and aromatics content, have been tested.

- EXPERIMENTAL PROCEDURE

Sixteen in-use mopeds and six 125 cm³ motorcycles, all about two years of age, have been tested over ECE 47 and ECE 40 driving cycles respectively. In figures 1 and 2 the speed profiles of two driving cycles are schematized.

All the two-wheelers tested were 2-stroke vehicles with fuelling control by carburetor and equipped with a separate oil tank for forced lubrication. No after-treatment device was installed at the exhaust.

Two-wheelers have been tested on chassis dynamometer test bench and the exhaust sampled by CVS system.

Diluted exhaust gases have been sampled in Tedlar bags and regulated emissions (CO, HC, NOx) and carbon dioxide, determined by conventional gas analyzers. Benzene, collected in bags, has been analyzed by gas-chromatography (GC). GC was a HP 5890 equipped with a FID detector and a gas inlet valve. The column was a Chromopack Al2O3/KCl (50 meter length, 0.53 mm internal diameter and 10m film thickness).

Fuel consumption has been generally evaluated by carbon mass balance and during two tests measuring it by graduated burette.

Since the mopeds tested were in-use vehicles some of them have been tampered by users. This practice is largely used in Italy to improve performance. This modification has the effect to increase the maximum speed and for this reason thirteen of the tested mopeds presented maximum speeds strongly exceeded the maximum speed allowed by regulation for this class of vehicle.

To characterize the real emission behavior of tampered mopeds they have been tested by driving standard ECE 47 cycle up to maximum on-road speed. In figure 3, maximum and minimum speed profiles, acquired during tests, are compared with the standard one.

Figure 3: Maximum and minimum speed profiles of tested mopeds

Figure 1: ECE 47 driving cycle

Both ECE 47 and ECE 40 driving cycles can be considered as warm cycles concerning emissions; in fact the sampling starts after half a cycle and one third a cycle respectively.

Figure 2: ECE 40 driving cycle

Each test has been repeated twice with each gasoline formulation.

The effect of lube oil on emissions has not been evaluated.

- FUELS

Two different gasoline formulations have been utilized. One gasoline was a commercial Italian leaded gasoline with typical market average values for benzene and aromatic content (1% and 33% respectively - named FUEL A). The other one  was a leaded gasoline with a lower benzene and aromatic content (0.7% and 28% respectively - named FUEL B) explicitly formulated to investigate the effect of those parameters on two-wheelers emissions, benzene and hydrocarbons in particular. Main fuel characteristics are reported in table 3.

Present Italian gasoline quality legislation is one of the most advanced in EU, having introduced in July 1998 1% vol. and 40% vol. max limits, for benzene and aromatic content respectively, well in advance to similar European mandatory limits on gasoline quality in place by 01.01.2000.

Low benzene – low aromatic gasoline has also introduced in Scandinavian countries (on incentivated basis) and in the US with the Reformulated Gasoline Program.

The choice of testing leaded gasolines is due to the fact that this grade is widely used today by two-wheelers.

|  | FUEL A | FUEL B |
| --- | --- | --- |
| Density @ 15°C (g/l) | 744.3 | 729.4 |
| RVP (kPa) | 69.5 | 75.2 |
| Benzene (%v/v) | 0.98 | 0.70 |
| Aromatics (%v/v) | 33.2 | 28.1 |
| Oxygen (%w/w) | <0.1 | <0.1 |
| Lead (g/l) | 0.138 | 0.146 |
| RON | 98.0 | 97.8 |
| MON | 87.4 | 87.5 |
| Distillation: |  |  |
| IBP (°C) | 32 | 31 |
| E70 (%v/v) | 37.5 | 38 |
| E100 (%v/v) | 48.5 | 57.5 |
| E180 (%v/v) | 95 | 95.5 |
| FBP (°C) | 213 | 212 |

Table 3: Gasoline characteristics

- EXPERIMENTAL RESULTS

- MOPEDS

The results are reported as mean values of two test repetitions. For each pollutant, the percentage variation respect to mean value has been always calculated lower than 5%.

The mean values are diagrammed in figures 4-9 for each species and fuel economy for the different mopeds (from 1 to 16) by varying the fuel (A and B).

CO values vary between 6-25 g/km and hence they result higher than EURO 1 limits. The emission of NOx are low, as expected from this engine technology due to the high degree of internal EGR. In fact the mopeds regulation imposes limit for the sum of HC and NOx and not for the separate NOx. As shown in figure 8, HC present values ranging between 4-14 g/km and no mopeds respect EURO 1 limits. CO2 has been measured between 25-40 g/km (figure 6). No effect of different gasoline composition has been evaluated on regulated pollutants.

Fuel economy has been calculated by carbon mass balance. The relative contributes of CO, CO2 and HC is almost comparable unlike passenger cars where CO2 plays the predominant role in fuel consumption calculation.

Fuel consumption for 100 km has been estimated in 2.5-5.0 liters, as shown in figure 7. For two mopeds the fuel consumption has been evaluated by direct measurements. In both tests the measured fuel consumption is greater than calculated one, with a discrepancy varying between 10-15%.

Benzene emissions vary from 41-150 mg/km when FUEL A is fed (table 4). Benzene contributes about 1.0-1.5 % of total hydrocarbon emissions by using this gasoline. When FUEL B is used benzene values decrease to 25-127 mg/km and its contribute to total HC is 0.7-1.0%.

In table 4 the percentage reduction of benzene emissions has been calculated for each mopeds by varying gasoline (A and B). Fuel B contains about -30% benzene and -15% aromatics than Fuel A. A mean reduction of about -23% has been observed when the low benzene content gasoline (FUEL B) is utilized in ECE47 tests.

In this initial experimentation all mopeds have been driven, during ECE 47 cycle, up to their maximum on-road speed because of their in real use emission factors were under consideration. No evaluation has been performed on the effect of bike modification with respect to standard cycle.

| mopeds | Benzene (mg/km) |  | % reduction |
| --- | --- | --- | --- |
|  | fuel A | fuel B |  |
| 1 | 106 | 69 | -34.9 |
| 2 | 119 | 111 | -6.7 |
| 3 | 150 | 102 | -32.0 |
| 4 | 143 | 91 | -36.4 |
| 5 | 113 | 97 | -14.2 |
| 6 | 41 | 25 | -39.0 |
| 7 | 113 | 89 | -21.2 |
| 8 | 96 | 78 | -18.7 |
| 9 | 84 | 71 | -15.5 |
| 10 | 147 | 127 | -13.6 |
| 11 | 124 | 88 | -29.0 |
| 12 | 69 | 50 | -27.5 |
| 13 | 147 | 123 | -16.3 |
| 14 | 110 | 84 | -23.6 |
| 15 | 71 | 50 | -29.6 |
| 16 | 143 | 122 | -14.7 |

Table 4: Benzene emission factors for mopeds

- MOTORCYCLES

In figures 10-15 the results obtained for each pollutant and fuel economy are reported for the different motorcycles (from 1 to 6) by varying the fuel (A and B). Also in this case, the results are reported as mean values of two test repetitions and the percentage variations are lower than 10% respect to mean values. Motorcycles emit, during ECE 40 cycle, 20-35 g/km of carbon monoxide, except motorcycle 5 that presents less than 10 g/km (Figure. 10). NOx vary between 0.020 and 0.040 g/km as schematized in figure 11. Carbon dioxide shows values 30-50 g/km (Figure.12) and total HC between 7-25 g/km (figure. 14). Fuels quality doesn’t seem influence CO, HC and NOx emissions.

Fuel consumption has been calculated in 3.5-7 liter to cover 100 km (figure. 13).

Benzene determination has shown emission levels vary from 99-350 mg/km when gasoline A is fueled (table 5). Benzene gives an about 1.3-1.4 % contribute of total hydrocarbons emissions by using this gasoline. When gasoline B is fueled benzene values decrease to 81-270 mg/km and it contributes to total HC around 1.0-1.1%. A mean reduction of about 26% has been observed when the low benzene content fuel (FUEL B) is utilized during ECE 40 cycles.

| 125 cc motorcycles | Benzene (mg/km) |  | % reduction |
| --- | --- | --- | --- |
|  | fuel A | fuel B |  |
| 1 | 180 | 131 | -27.2 |
| 2 | 330 | 270 | -18.2 |
| 3 | 337 | 262 | -22.2 |
| 4 | 329 | 183 | -44.4 |
| 5 | 99 | 81 | -18.2 |
| 6 | 350 | 263 | -24.8 |

Table 5: Benzene emission factors for 125 cc motorcycles

DISCUSSION

Mean emission and consumption factors that we have measured for mopeds are in perfect agreement with factors utilized by Copert II [4]. In table 6 the above factors are reported for comparison.

| Mopeds | CO (g/km) | NOx (g/km) | HC (g/km) | Fuel Consumption (g/km) |
| --- | --- | --- | --- | --- |
| Experimental results | 17.6 | 0.03 | 9.21 | 27.8 |
| COPERT II | 15.0 | 0.03 | 9.00 | 25.0 |

Table 6: Comparison of emission and consumption factors for mopeds

These emission factors are mean values evaluated during the tests with all sixteen mopeds. But a great variability has been found in regulated emissions values by changing the tested moped. This can be justified by the different mopeds maintenance and engines regulation.

An analogous comparison can be done for the results obtained by driving the six 125 cc motorcycles. The Copert II methodology, for this class of vehicles, makes use of general relationships between mean speed and emission or consumption factors. In table 7 the mean results obtained in this work are compared with Copert II ones by using the general expressions at a mean speed of 19 km/h (this is mean speed of ECE 40 driving cycle).

Our experimental results are in a good agreement with those calculated by Copert II relationships, for this two-wheelers typology too. It’s necessary to point out that Copert doesn’t take engine displacement into consideration, in fact it considers all the motorcycles over 50 cc.

| 125 cc | CO (g/km) | NOx (g/km) | HC (g/km) | Fuel Consumption (g/km) |
| --- | --- | --- | --- | --- |
| Experimental results | 23.2 | 0.03 | 19.10 | 43.0 |
| COPERT II | 21.0 | 0.04 | 13.60 | 35.2 |

Table 7: Comparison of emission and consumption factors for motorcycles

Benzene exhaust levels reflect the benzene content in two fuels for both of two-wheelers typologies. It is well known that benzene emissions are due to unburned benzene of fuel and to that produced during the combustion process, mainly by combustion of aromatic hydrocarbons [5]. Since FUEL B is composed by a lower content of aromatics and benzene than FUEL A, the experimental results are in agreement with a such consideration. These benzene emission levels find correspondence to HBEFA benzene factors [6]. In HBEFA database benzene is estimated about 100-125 mg/km for mopeds and 300-500 mg/km for motorcycles.

- Conclusions

The experimental work, object of this paper, has allowed to collect information regarding the emissions of in-use motorcycles, in particular with reference to benzene emission levels.

The results of this experimental work show regulated emissions and fuel consumption factors of mopeds and 125 cc motorcycles in agreement with factors adopted in Copert II methodology.

Benzene emissions result in the order of 50-350 mg/km and they give an about 1.0-1.5 % contribute of total hydrocarbon emissions by using a commercial gasoline with 1% benzene content. When a lower benzene (0.7%) and aromatics content gasoline is fueled benzene values decrease to 25-270 mg/km and it contributes to total HC around 0.7-1.1%. These benzene emission levels are comparable with HBEFA benzene factors [6].

An average benzene reduction of about 23-26% has been observed when the low benzene content fuel (FUEL B) is utilized during ECE 40 and ECE 47 driving cycles. This effect, being in the same range of the reduction in benzene content of the fuel, seems to confirm that benzene (and hydrocarbons) emitted by two-stroke engine derive mainly by unburnt fuel which escapes the combustion process.

These first experimental results are set in a wide research project which has the aim to evaluate vehicle emissions in real driving conditions. In particular the application of ECE 40 and 47 driving cycles to two-wheelers presents the shortcoming to underestimate the cycle dynamics. Moreover this procedure does not take into consideration the warming-up behavior of motorcycles. Another open question is the evaluation of other unregulated emissions at two-wheelers exhaust, such as particulate matter, 1-3 butadiene [5,7]

In the next  few years, conventional two-stroke motorcycles equipped with carburetor will be progressively substituted in Europe by vehicles which meet new emission standard, as two-stroke motorcycles with direct injection, which might satisfy the EURO 2 emission limits even without catalyst [2,8].

- Acknowledgments

A sincere thanks to the whole staff of Istituto Motori Emission Laboratory and in particular to Francesco Vitale and Domenico Sirico for their availability and collaboration.

CONTACT

eng. Maria Vittoria Prati, researcher of Istituto Motori CNR.

Istituto Motori CNR - viale Marconi, 8  Napoli 80125 Italy

phone: 0039-081-7177210, fax: 0039-081-2396097, e-mail:mavi@motori.im.na.cnr.it

- References

- R.C. Rijkeboer “The motorcycle emission situation. A report to the European Commission” 1999 TNO Report 99.OR.VM.035.1/99

- M. Nuti “Emissions from Two-Stroke Engines” 1998 SAE ISBN 0-7680-0125-X

- European Commission “Auto-Oil II Cost-effectiveness Study” Transport base case report, EU 1999

- European Environment Agency “Computer Programme to Calculate Emissions from Road Transport - COPERT II” 2nd Edition, 1997

- G. Marchesi, G. Martini, R. Mercogliano “Fuel Quality and Emission Performance of Two-stroke and Four-stroke S.I. Engine” International Conference Power Two Wheels, Pisa Italy, 1998

- UBA, SAEFL “Handbook Emission Factors for Road Transport”, 1999 Version 1.2

- M. Walters “Two-stroke motorcycle use in southern Italy: externalities, relevant legal regimes and policy proposals” Dissertation of University of London, Wye College, 1997

- M. Nuti, R. Pardini, D. Caponi “FAST Injection System: PIAGGIO Solution for ULEV 2T SI Engines” SAE Paper 9703620

- Definitions, Acronyms, Abbreviations

IBP	Initial Boiling Point

FBP	Final Boiling Point

RVP	Reid Vapor Pressure

CVS	Constant Volume Sampler

GC	Gas Chromatography

FID	Flame Ionization Detector

EGR	Exhaust Gas Recirculation

Copert 	Computer Programme to Calculate Emissions from Road Transport

MOPEDS (50 cc)

Figure 4: CO emission values

Figure 5: NOx emission values

Figure 6: CO2 emission values

Figure 7: Estimated fuel economy

Figure 8: HC emission values

Figure 9: Benzene emission values

MOTORCYCLES (125 cc)

Figure 10: CO emission values

Figure 11: NOx emission values

Figure 12: CO2 emission values

Figure 13: Estimated fuel economy

Figure 14: HC emission values

Figure 15: Benzene emission values