# Evaluation of Benzo(a)Pyrene emissions from cokery of an iron plant in Servola quarter, Trieste

A study on Benzo[a]Pyrene (B[a]P) emissive impacts over the urban area next to Trieste industrial area has been carried out in consideration of the criticality due to the presence of a built up area adjoining a siderurgical plant. The study employed a modellistic chain combining both emissive data from the cokery  of an iron plant in Servola quarter, Trieste, estimated by forensic consultants, and the meteorological parameters responsible of transportation/diffusion of B[a]P.

The modellistic technique was based on a merely dispersive model and it was calibrated through measures on two monitoring stations in Servola quarter (IT1475A station and an industrial station called RFI). In particular, for the numerical simulation, it was assumed that all B[a]P comes only from the cokery plant; since B[a]P emissions are of fugitive type, due to the cokery managing way, it was assumed a constant hourly emission all over the indicative year, on the basis of plant engineering considerations.

The aim of numerical simulations was identifying the exceedence area of 1 ng/Nm3, fixed by law in force as target value for human health, and finding possible variations in emissive features of Servola siderurgical plant all over the year 2012.

The outcome of this evaluation technique model-monitoring combinined is shown in Fig. 1.

The observed shape in pollutant distribution can be explained considering that the examined zone is prone to breezes wind regimen: in the morning time, breeze blows from North East to South West allowing pollutants dispersion from inland to the sea, while during the warmer periods of the day – and more patently during the warmer seasons, it blows from North West to South East. This afternoon breeze effect is considered the main cause of the South East exceedence area, modulated by the area orography.

Mean B[a]P concentrations along North East- South West directrix are relatively low, as expected, since wind usually blowing along this directrix is usually quite strong and it can dilute pollutants enaugh to reach low concentration levels.

Observing the time variation of over 1 ng/Nm3 areas during the 2012 six couples of months, the area characterized by 1 ng/Nm3 or higer B[a]P concentrations shows to reduce remarkably from the first four to the last two couples of months. On the basis of the available information, coming both from air quality monitoring network and numerical simulations, this concentrations reduction seem to be due not only to meteorological peculiarities but mainly to an actual emissions reduction from cokery.

It can be deduced from the numerically estimated and measured B[a]P concentrations ratio. In particular, the observed ratio between expected (with emission estimated to be constant during the whole year) and measured (at the industrial station RFI) concentration was less than half from the first four couples of months to the last two.

These hypothesis have been confirmed observing the trend of daily benzene (another cokery plants typical pollutant) levels registered from the month of March 2007 to March 2013 (Fig. 2)

Fig. 1 The pink area represent the indicative zone of B[a]P target value exceedence on the year 2012 in the Servola quarter (Trieste), obtained both by numerical simulations and punctual measures of this pollutant; the light blue area shows the exceedence area during the last two months of the same year.

Figure 2. Trend of daily mean benzene concentrations from March 2007 to March 2013 in the monitoring stations set in the study area. PIT: IT1475A; MUG: IT1477A; SVE: IT1476A; SAB: IT1651A; CAR: IT1090A

# BIBLIOGRAPHY

- http://www.arpa.fvg.it/export/sites/default/tema/aria/utilita/Documenti_e_presentazioni/tecnico_scientifiche_docs/Relazione_qualita_aria_ts_2012.pdf